Ultra HD Video Scaler

Model No. LY1601

- Support HDMI2.0/HDCP2.2/18Gbps

- 4K UHD down-scale to 1080p.

- 1080p up-scale to 4K.

- Audio extracted and output with 3.5mm Jack and Toslink.

Warning

- Do not expose this box to Rain, Moisture, and Dripping.

- Only use accessories specified by the manufacture.

- Unplug this box during Lightning Storms.

1. Introduction

LY1601 Ultra HD Video Scaler box supports up-scale 1080p to 4K, down-scale 4K to 1080p.

For non-4K and non-1080p, it will bypass input to output.

It’s able to extracts audio from HDMI input and output with 3.5mm Jack and Toslink.

2. Front Panel, Buttons and LEDs

OUTPUT button: select output resolution between PASS, 4K, 1080P.

*1. PASS: bypass input with any formats to output.

*2. 4K: only works with 1080p input. It will up-scale 1080p to 4K.

*3. 1080p: only works with 4K input. It will down-scale 4K to 1080p.

*4. Frame rate conversion is not supported.

EDID button: select AUTO, 4K, 1080P

*1. AUTO: It will automatically copy the EDID data from display devices.

*2. 4K: Force the EDID to 4K.

*3. 1080p: Force the EDID to 1080p.

3. Back Panel

*1. RS232 Phoenix Connector is only for firmware upgrade

4. Specifications

| Operating Temperature Range | 0 ºC ~ 40ºC |
| --- | --- |
| Storage Temperature | -20 ºC ~ 60ºC |
| Input / Output HDMI Format | Up to HDMI 2.0/HDCP2.2/18Gbps, 4:4:4 60Hz. |
| Power | 3watts (Max), 5V/1.5A power adapter |
| Main Unit Dimensions (L*H*W) | 143mm×83mm×26mm |
| Main Unit Weight | 0.35kg |
| Package Size (L*H*W) | 220mm×196mm×54mm |

5. Package Contents

- LY1601 Ultra-HD Video Scaler			1pcs

- User Manual							1pcs

- Power Supply, 5V/1.5A 				1pcs

6. Appendix, LY1601 input/output resolution

| Scale up |  | Scale down |  |
| --- | --- | --- | --- |
| In | Out | In | Out |
| 1080p24 | 3840x2160/24 | 3840x2160/24 | 1080p24 |
| 1080p25 | 3840x2160/25 | 3840x2160/25 | 1080p25 |
| 1080p30 | 3840x2160/30 | 3840x2160/30 | 1080p30 |
| 1080p50 | 3840x2160/50 | 3840x2160/50 | 1080p50 |
| 1080p60 | 3840x2160/60 | 3840x2160/60 | 1080p60 |
|  |  | 4096x2160/24 | 1080p24 |
|  |  | 4096x2160/25 | 1080p25 |
|  |  | 4096x2160/30 | 1080p30 |
|  |  | 4096x2160/50 | 1080p50 |
|  |  | 4096x2160/60 | 1080p60 |

| Resolution supported for passthrough |  |  |  |
| --- | --- | --- | --- |
| 3840x2160/24 | 480i | 1080P30 | 1360*768/60/75 |
| 3840x2160/25 | 576i | 1080P50 | 1400*1050/60/60RB |
| 3840x2160/30 | 480p | 1080P60 | 1440*900/60/60RB |
| 3840x2160/50 | 576p | 640*480/75 | 1680*1050/60 |
| 3840x2160/60 | 720p50 | 800*600/60/75 | 1920*1200/60RB |
| 4096x2160/24 | 720p60 | 1024*768/60/75 | 1600*900/60RB |
| 4096x2160/25 | 1080i50 | 1280*768/60/75 |  |
| 4096x2160/30 | 1080i60 | 1280*800/60/75 |  |
| 4096x2160/50 | 1080P24 | 1280*960/60/75 |  |
| 4096x2160/60 | 1080P25 | 1280*1024/60/75 |  |